JJ IS40BXPA Setup and Integration Guide

English Product Documentation

Hardware specifications, ACServer deployment, configuration and data interface reference for the JJ-IS40BXPA Bluetooth AoA gateway.

Jinju Intelligence (Beijing) Technology Co., Ltd. | zydc0915@gmail.com

Figures retain the original hardware labels and software interface. The accompanying English instructions identify the controls and installation steps.

JJ-IS40BXPA Bluetooth Gateway

Product Specifications and User Manual

1 Copyright and Notices

Copyright. All rights reserved.

Confidentiality: This document contains confidential information for the designated recipient only and must not be disclosed to third parties.

Disclaimer: No express or implied representation or warranty is made in this document. No responsibility is accepted for merchantability or fitness for a particular purpose.

2 Hardware Specifications

2.1 Product Overview

The JJ-IS40BXPA is a Bluetooth 5.1 AoA (Angle of Arrival) indoor ceiling gateway. Its on-board antenna array is designed for precise indoor positioning, with accuracy within 0.5 m in unobstructed conditions. A dual-core ARM M4F processor, PoE and DC power options support healthcare, warehousing, personnel and asset tracking applications.

2.2 Core Technical Specifications

2.2.1 Processor and Memory

Main MCU: Dual-core ARM M4F at 64 MHz

Flash： 1MB + 512KB

RAM： 256KB + 128KB

2.2.2 RF and Positioning Performance

Bluetooth protocol: Bluetooth 5.1 / 4.2, backward compatible

Operating frequency: 2.4 GHz

Transmit power: 10 dBm

Receiver sensitivity: > -95 dBm

Positioning accuracy: < 0.5 m in unobstructed conditions

Coverage: Radius approximately 1.5 times the installation height

Antenna: On-board antenna array

2.2.3 Electrical Specifications and Interfaces

Power supply: PoE or DC 5 V / 1 A

Rated power: 3 W

Network interface: 10/100 Mbps auto-negotiating LAN

Network protocols: UDP, DHCP / static IP

2.2.4 Physical and Environmental Specifications

Dimensions: 230 x 230 x 40 mm

Weight: 500 g

Housing material: ABS

Operating temperature: -20 to 60 degrees C

Operating humidity: 0 to 95%, non-condensing

2.3 Product Appearance

3 ACServer Software

3.1 Software Overview

ACServer is the positioning engine for this system. It supports Windows 7 and later and Linux distributions. It manages AoA gateways, processes raw azimuth and elevation data, calculates target positions and pushes results to application platforms through standard interfaces.

3.2 Main Functions

Gateway management: Online configuration, parameter readback, status monitoring with online/offline notifications, and traffic statistics.

Live tag monitoring: View positioning-tag battery levels, online status and current locations.

Positioning engine: Receive raw gateway observations, optimize the calculations and output final coordinates.

Data push: Socket (TCP/UDP), WebSocket, MQTT and HTTP interfaces are supported.

4 Deployment and Configuration

4.1 Hardware Connection and Network Access

Power connection

PoE: Connect one end of the Ethernet cable to a PoE switch and the other to the gateway.

DC: Connect a 5 V / 1 A power adapter, then connect the Ethernet cable.

IP address configuration

The gateway uses DHCP by default. On a DHCP-enabled LAN, it acquires its network configuration after power-up.

For static IP configuration, use the WeChat mini program named Smart Bluetooth Gateway Tool and enable Bluetooth permissions on the phone. The original tool uses a Chinese interface.

Keep the gateway and ACServer computer on the same subnet. For tests on the same LAN, the destination address can remain 0.0.0.0 (broadcast mode), allowing ACServer to discover the gateway. For a remote subnet or cloud server, set the destination to the server's public IP address.

4.2 Starting ACServer

Windows: The source procedure instructs the operator to disable the firewall and double-click start.bat. Deployment note: use a controlled test network; in production, configure the required firewall rules instead of disabling protection globally.

Linux

Foreground: ./acserver -v=7 -alsologtostderr

Background: nohup ./acserver -v=7 -alsologtostderr &

Management login

Open http://[server-IP]:8888/v/user/login.html in a browser.

Default username: admin. Default password: 123.

4.3 System Parameter Configuration

Important: Complete the following configuration after the first login to establish system communications.

Open System Parameter Configuration.

Set Local IP to the actual IP address of the computer or server running ACServer.

Select the ACServer network type: LAN or public network, as appropriate.

Save the settings, then close and restart ACServer for the changes to take effect.

4.4 Data Push

AoA tag data push

{
    "Gateway":"10000060",
    "NodeId":"00000000",

"Group":"group1",
    "SystemId":"10990145",
    "Type":"AOA_data",
    "conx":-0.8987940462991669,
    "ka":206,
    "ke":42,
    "mac":"19,00,00,00,60,C2",
    "rssi":8,
    "siny":-0.43837114678907746,
    "tanh":0.9004040442978399,
    "x":-0.5547684731158157,
    "y":-0.27453919672639904
}

| Parameter | Type | Description |
| --- | --- | --- |
| Gateway | String | ID of the AoA gateway uploading this record |
| NodeId | String | Currently unused; replaced by mac |
| SystemId | String | ACServer ID identifying the uploading instance when multiple servers are deployed |
| Type | String | Message type; a fixed value for this message |
| mac | String | Unique identifier of the tag |
| rssi | Int | Tag signal strength |
| Ka | Int | Raw AoA azimuth value |
| ke | Int | Raw AoA elevation value |
| conx | Float | Raw X-direction value calculated at a unit distance of 1 m |
| siny | Float | Raw Y-direction value calculated at a unit distance of 1 m |
| tanh | Float | Distance from the gateway calculated for a height of 1 m |
| x | Float | Final calculated X coordinate |
| Y | Float | Final calculated Y coordinate |