DIN Rail Bluetooth 4G Gateway

English Product Documentation

A metal enclosure, external antenna and wide-input DC or PoE power suit industrial Bluetooth data collection. UDP/MQTT transport and OTA maintenance support integration with existing systems.

Jinju Intelligence (Beijing) Technology Co., Ltd. | zydc0915@gmail.com

Selection Notes

The document lists a 4G antenna interface, but the network table specifies LAN. Confirm the 4G backhaul module, configuration and connectivity service when ordering.

IS40B4GDP Series Bluetooth Gateway

Product Specifications

Important Notices

No Warranty

No express or implied representation or warranty is made concerning this document. No liability is accepted for merchantability, fitness for a particular purpose, or indirect, special or consequential loss.

Confidentiality

This document, including its attachments, contains confidential information. The recipient acknowledges its confidential nature and may use it only for the specified purpose. It must not be disclosed to any third party.

Chapter 1 IS40B4GDP DIN Rail Bluetooth 4G Gateway

1.1 Electrical Specifications

| Power supply | PoE or DC 12 to 36 V |
| --- | --- |
| Power | 5W |
| Operating temperature | -20℃—60℃ |
| Operating humidity | 0 to 95%, non-condensing |

1.2 Technical Specifications

| Main MCU | 32-bit ARM M4 |
| --- | --- |
| Operating frequency | 2.4GHz |
| Transmit power | Bluetooth: 10 dBm |
| Protocol | Bluetooth 5.0 |
| Receiver sensitivity | BLE:>-104dBm |
| Transmission range* | BLE：300m |
| Encryption | AES encryption supported |

Note: Transmission range is specified for line-of-sight conditions.

1.3 Physical Specifications

| Dimensions | Mounting ears folded: 185 x 140 x 30 mm; extended: 210 x 140 x 30 mm. Antenna height: 230 mm. |
| --- | --- |
| Color | Black |
| Weight | 580g |
| Material | Sheet metal |
| Ingress protection | None |

1.4 Additional Specifications

| Network connection | 10/100 Mbps LAN port |
| --- | --- |
| Network addressing | DHCP or static IP |
| Network transport | UDP/MQTT |
| Antenna | External fiberglass antenna |
| Antenna connector | N-type female |

1.5 Configuration Functions

| Destination IP | Domain name or IP address supported |
| --- | --- |
| Over-the-air update | OTA supported |
| Configuration method | WeChat mini program or ACServer |

| Bluetooth antenna connector | N-type |
| --- | --- |

1.6 Antenna Connectors

| 4G antenna connector | N-type |
| --- | --- |

1.7 Product Appearance