iBeacon Beacon Series

English Product Documentation

Indoor, industrial-protection and wristband variants support standard iBeacon broadcasts. Configure UUID, Major, Minor, interval and transmit power by phone.

Jinju Intelligence (Beijing) Technology Co., Ltd. | zydc0915@gmail.com

Selection Notes

The source lists up to ten years for indoor/industrial variants and three to six months for the wristband; battery, broadcast configuration and temperature affect endurance. Confirm explosion-protection certificates and scope when selecting a model.

JJ-IB Series iBeacon Specifications

Jinju Intelligence Beijing Technology Co Ltd - Haidian Beijing

Date: 19 September 2024

Product enquiries and technical support: zydc0915@gmail.com

Revision History

| Version | Changes | Author | Reviewer | Date |
| --- | --- | --- | --- | --- |
| V1.0.0 | Initial release |  |  | 19.11.15 |
| V1.0.1 | Added wristband iBeacon |  |  | 20.06.02 |

Important Notices

Copyright

Copyright 2019. All rights reserved.

No Warranty

No express or implied representation or warranty is made concerning this document. No liability is accepted for merchantability, fitness for a particular purpose, or indirect, special or consequential loss.

Confidentiality

This document, including its attachments, contains confidential information. The recipient acknowledges its confidential nature and may use it only for the specified purpose. It must not be disclosed to any third party.

JJ-IB32I Indoor Beacon

Electrical Specifications

| Battery capacity | 2400mAh |
| --- | --- |
| Peak current | 5.5mA |
| Sleep current | <1uA |
| Average current | <30uA* |
| Operating temperature | -20℃—60℃ |
| Operating humidity | 0 to 95%, non-condensing |

Average current depends on the broadcast interval and power. The stated default is 500 ms at 0 dBm.

Operating frequency

| Bluetooth frequency | 2.4~2.485GHz |
| --- | --- |

Physical Specifications

| Dimensions | 53*23mm |
| --- | --- |
| Color | White |
| Weight | 45g |
| Housing material | ABS |

Note: Physical specifications vary with the packaging variant.

Colors can be selected for bulk orders; stocked units ship according to availability.

Weight varies slightly by model.

Additional Specifications

| Transmit power | -20 dB to 4 dB in 4 dB steps |
| --- | --- |
| Coverage | Maximum diameter: 100 m |
| Battery life | Up to 10 years* |
| Tamper detection | Limit-switch tamper detection* |
| Water resistance | IP55 |
| Security | AES encryption, tamper protection, connectable and non-connectable modes |
| Supported protocols | Apple standard iBeacon protocol; Bluetooth 5.0 |

*

Battery life depends on transmit power, transmission interval and ambient temperature.

Optional function; not included in the standard configuration.

Configuration Functions

| Phone configuration | Configure UUID, Major, Minor, broadcast interval, transmit power and tamper-protection key. |
| --- | --- |
| Over-the-air update | OTA supported |

Product Drawings

Packaging Specifications

48 units per carton

JJ-IB100 Explosion Protected Beacon

Electrical Specifications

| Battery capacity | 2600 mAh; optional 5200 mAh. Default: 2600 mAh. |
| --- | --- |
| Peak current | 5.5mA |
| Sleep current | <1uA |
| Average current | <30uA* |
| Operating temperature | -20℃—60℃ |
| Operating humidity | 0 to 95%, non-condensing |

Average current depends on the broadcast interval and power. The stated default is 500 ms at 0 dBm.

Operating frequency

| Bluetooth frequency | 2.4~2.485GHz |
| --- | --- |

Physical Specifications

| Dimensions | 120*120*24mm |
| --- | --- |
| Color | Industrial yellow, black, blue, off-white and other optional colors |
| Weight | 185-195g |
| Housing material | Modified ABS |

Notes

Colors can be selected for bulk orders; stocked units ship according to availability.

Additional Specifications

| Transmit power | -40dB-4dB |
| --- | --- |
| Coverage | Maximum diameter: 100 m |
| Battery life | Up to 10 years with maximum battery capacity |
| Water resistance | IP68 |
| Compression resistance | 10 tonnes |
| Certification | Explosion-protection certification |
| Security | AES encryption, tamper protection, connectable and non-connectable modes |
| Supported protocols | Apple standard iBeacon protocol; Bluetooth 5.0 |

Configuration Functions

| Phone configuration | Configure UUID, Major, Minor, broadcast interval, transmit power and tamper-protection key. |
| --- | --- |
| Over-the-air update | OTA supported |

Product Images

Packaging Specifications

60 units per carton

JJ-IB32W Wristband iBeacon

Electrical Specifications

| Battery capacity | 50mAh |
| --- | --- |
| Power supply | Rechargeable lithium battery |
| Charging method | Mirco usb |
| Charging voltage | 5V |
| Peak current | 5.5mA |
| Sleep current | <1uA |
| Average current | <30uA* |
| Operating temperature | -20℃—60℃ |
| Operating humidity | 0 to 95%, non-condensing |

Average current depends on the broadcast interval and power. The stated default is 500 ms at 0 dBm.

Operating frequency

| Bluetooth frequency | 2.4~2.485GHz |
| --- | --- |

Physical Specifications

| Dimensions | 250*40*11mm |
| --- | --- |
| Color | White, pink, blue, black, orange or green |
| Weight | 30g |
| Housing material | Plastic |

Note: Colors can be selected for bulk orders; stocked units ship according to availability.

Additional Specifications

| Transmit power | -40dB-4dB |
| --- | --- |
| Coverage | Maximum diameter: 100 m |
| Battery life | 3 to 6 months |
| Water resistance | IP53 |
| Security | AES encryption, tamper protection, connectable and non-connectable modes |
| Supported protocols | Apple standard iBeacon protocol; Bluetooth 5.0 |

Configuration Functions

| Phone configuration | Configure UUID, Major, Minor, broadcast interval, transmit power and tamper-protection key. |
| --- | --- |
| Over-the-air update | OTA supported |

Product Images