Bluetooth UWB Asset Tag

English Product Documentation

Bluetooth and UWB connectivity combine with a three-axis sensor and filtering. Compatible UWB gateways enable asset tracking, with motion detection, low-battery alerts and batch updates.

Jinju Intelligence (Beijing) Technology Co., Ltd. | zydc0915@gmail.com

Selection Notes

Maximum stationary standby is more than three years, not continuous tracking endurance. Confirm the hardware, protocol and firmware combination for TDoA or PDoA operation.

JJ-IPBU Bluetooth UWB Asset Location Tag

Product Specifications

Jinju Intelligence Beijing Technology Co Ltd - Haidian Beijing

Date: 29 August 2025

Product enquiries and technical support: zydc0915@gmail.com

Revision History

| Version | Changes | Author | Reviewer | Date |
| --- | --- | --- | --- | --- |
| V1.0.0 | Initial release |  |  | 25.10.01 |

Important Notices

Copyright

Copyright 2021 Jinju Intelligence Beijing Technology Co Ltd. All rights reserved.

No Warranty

No express or implied representation or warranty is made concerning this document. No liability is accepted for merchantability, fitness for a particular purpose, or indirect, special or consequential loss.

Confidentiality

This document, including its attachments, contains confidential information. The recipient acknowledges its confidential nature and may use it only for the specified purpose. It must not be disclosed to any third party.

Chapter 1 JJ-IP32BU Asset Location Tag Specifications

1.1 Functions

A built-in three-axis sensor dynamically adjusts broadcast and positioning frequency. Broadcast settings can be configured with a mini program. The high-capacity battery provides more than three years of maximum stationary standby. An integrated filtering algorithm supports positioning accuracy down to 0.3 m with a compatible UWB gateway. Optional buzzer and tamper-detection components enable asset finding and tamper alarms.

1.2 Electrical Specifications

| Operating voltage | 3.6V |
| --- | --- |
| Battery specification | 1 x power-type 14505 cell |
| Transmit power | BLE：8dBm UWB：-10dBm |
| Protocol | BLE5.0 802.15.4 |
| Receiver sensitivity | BLE:<-101dBm UWB: <-110dBm |
| UWB channels | 5, 9 |
| UWB data rates | 110 kbps, 850 kbps, 6.8 Mbps |
| Maximum current | <130mA@3.3V |
| Listening current | <6Ma@3.3v |
| Sleep current | <30Ua@3.3V |
| Operating temperature | -20℃—60℃ |
| Operating humidity | 0 to 95%, non-condensing |

1.3 Operating Frequencies

| UWB frequency | 6000MHz~9000MHz |
| --- | --- |
| Bluetooth frequency | 2.4~2.485GHz |

1.4 Physical Specifications

| Dimensions | 81*41*20mm |
| --- | --- |
| Color | White |
| Weight | 100g |
| Housing material | PC |

1.5 Additional Specifications

| Motion and stationary detection | Supported |
| --- | --- |
| Low-battery notification | Supported |
| Firmware update | Individual updates by phone or batch updates through a gateway |

1.6 Configuration Functions

| Beacon filtering | Supported |
| --- | --- |
| Data transmission interval | Adjustable |
| Motion amplitude trigger | Adjustable sensitivity |
| Data transmission after becoming stationary | Supported |

Note: Configuration requires a phone supporting Bluetooth 4.2 or later.

1.7 Product Appearance