Bluetooth Asset Tag

English Product Documentation

A three-axis sensor adjusts broadcast and positioning frequency according to motion. Pair the tag with a Bluetooth gateway for asset positioning, low-battery monitoring and batch maintenance.

Jinju Intelligence (Beijing) Technology Co., Ltd. | zydc0915@gmail.com

Selection Notes

The source lists more than eight years of maximum stationary standby, not continuous tracking endurance. Validate performance with the actual broadcast policy and operating conditions.

JJ-IP Bluetooth Asset Location Tag

Product Specifications

Jinju Intelligence Beijing Technology Co Ltd - Haidian Beijing

Date: 29 August 2022

Product enquiries and technical support: zydc0915@gmail.com

Revision History

| Version | Changes | Author | Reviewer | Date |
| --- | --- | --- | --- | --- |
| V1.0.0 | Initial release |  |  | 21.10.01 |

Important Notices

Copyright

Copyright 2021 Jinju Intelligence Beijing Technology Co Ltd. All rights reserved.

No Warranty

No express or implied representation or warranty is made concerning this document. No liability is accepted for merchantability, fitness for a particular purpose, or indirect, special or consequential loss.

Confidentiality

This document, including its attachments, contains confidential information. The recipient acknowledges its confidential nature and may use it only for the specified purpose. It must not be disclosed to any third party.

Chapter 1 JJ-IP32B Asset Location Tag Specifications

1.1 Functions

A built-in three-axis sensor dynamically adjusts broadcast and positioning frequency. Broadcast settings can be configured with a mini program. The high-capacity battery provides more than eight years of maximum stationary standby. An integrated filtering algorithm supports 1 to 3 m positioning with a compatible Bluetooth gateway. Optional buzzer and tamper-detection components enable asset finding and tamper alarms.

1.2 Electrical Specifications

| Operating voltage | 3.6V |
| --- | --- |
| Battery specification | 1 x power-type 14505 cell |
| Maximum Bluetooth listening current | <6mA@3.6v |
| Maximum Bluetooth transmit current | <8.5mA@3.6v |
| Sleep current | <10uA@3.6V |
| Operating temperature | -20℃—60℃ |
| Operating humidity | 0 to 95%, non-condensing |

1.3 Operating Frequencies

| Bluetooth frequency | 2.4~2.485GHz |
| --- | --- |

1.4 Physical Specifications

| Dimensions | 81*41*20mm |
| --- | --- |
| Color | White |
| Weight | 100g |
| Housing material | PC |

1.5 Additional Specifications

| Motion and stationary detection | Supported |
| --- | --- |
| Low-battery notification | Supported |
| Firmware update | Individual updates by phone or batch updates through a gateway |

1.6 Configuration Functions

| Beacon filtering | Supported |
| --- | --- |
| Data transmission interval | Adjustable |
| Motion amplitude trigger | Adjustable sensitivity |
| Data transmission after becoming stationary | Supported |

Note: Configuration requires a phone supporting Bluetooth 4.2 or later.

1.7 Product Appearance