Indoor Bluetooth LoRa Gateway

English Product Documentation

An indoor metal gateway combining Bluetooth and LoRa collection. The SX1301 supports eight uplink channels and one downlink channel for personnel tags and sensor data.

Jinju Intelligence (Beijing) Technology Co., Ltd. | zydc0915@gmail.com

Selection Notes

Classified as indoor according to the filename. One source heading says outdoor, but the protection rating is listed as none; no waterproof capability is claimed. LoRa does not automatically mean LoRaWAN.

JJ-IS40BL Series Bluetooth LoRa Gateway

Product Specifications

Jinju Intelligence Beijing Technology Co Ltd - Haidian Beijing

Date: 19 December 2020

Product enquiries and technical support: zydc0915@gmail.com

Revision History

| Version | Changes | Author | Reviewer | Date |
| --- | --- | --- | --- | --- |
| V1.0.0 | JJ-IS40BL |  |  | 19.04.23 |

Important Notices

Copyright

Copyright 2019 Jinju Intelligence Beijing Technology Co Ltd. All rights reserved.

No Warranty

No express or implied representation or warranty is made concerning this document. No liability is accepted for merchantability, fitness for a particular purpose, or indirect, special or consequential loss.

Confidentiality

This document, including its attachments, contains confidential information. The recipient acknowledges its confidential nature and may use it only for the specified purpose. It must not be disclosed to any third party.

JJ-IS40BL Outdoor Gateway

Electrical Specifications

| Power supply | POE |
| --- | --- |
| Operating temperature | -20℃—60℃ |
| Operating humidity | 0 to 95%, non-condensing |

Technical Specifications

| Main MCU | 32-bit ARM M4 |
| --- | --- |
| Operating frequency | Bluetooth: 2.4 GHz; LoRa: 480 MHz |
| Transmit power | Bluetooth: 8 dBm; LoRa: 25 dBm |
| Protocol | Bluetooth 5.0; LoRa |
| Receiver sensitivity | BLE:<-101dBm LORA: <-138dBm |
| Transmission range* | BLE：300m ； LORA：1Km |
| Device capacity | BLE: More than 100 nodes scanned per second; LoRa: More than 20 nodes scanned per second |
| Encryption | AES encryption supported |
| LoRa channels | 8 uplink channels and 1 downlink channel |
| LoRa data rate | 0~37.5kbps |
| LoRa controller | Sx1301 |

Note: Transmission range is specified for line-of-sight conditions.

Physical Specifications

| Dimensions | Mounting ears folded: 185 x 140 x 30 mm; extended: 210 x 140 x 30 mm. Antenna height: 230 mm. |
| --- | --- |
| Color | Black |
| Weight | 600g |
| Material | Sheet metal |
| Ingress protection | None |

Additional Specifications

| Network connection | 10/100 Mbps LAN port |
| --- | --- |
| Network addressing | DHCP or static IP |
| Network transport | UDP |
| Antenna | External magnetic-base antenna |
| Antenna connector | SMA |

Configuration Functions

| Destination IP | Domain name or IP address supported |
| --- | --- |
| Over-the-air update | OTA supported |
| Configuration method | Bluetooth mini program or ACServer |

| Bluetooth antenna connector | SMA connector |
| --- | --- |

Antenna connector

Product Appearance

Front

Rear