Indoor UWB Positioning Station

English Product Documentation

A Bluetooth and UWB station for factories, healthcare and offices, with support for two-dimensional systems and deployment in three-dimensional warehouse applications. Ceiling installation and PoE simplify cabling.

Jinju Intelligence (Beijing) Technology Co., Ltd. | zydc0915@gmail.com

Selection Notes

The source describes accuracy both as down to 30cm and greater than 30cm, so no guaranteed accuracy is stated here. Confirm frequency, channel and TDoA/PDoA firmware support.

JJ-U Indoor UWB Positioning Station

1.1 Product Description

The UWB station supports two- and three-dimensional IoT positioning applications, half-duplex and full-duplex communication, and 10/100 Mbps Ethernet connectivity to a cloud server. Integrated Bluetooth and UWB baseband chips provide UWB data transfer, planar positioning and ranging. Bluetooth/UWB dual-mode communication serves healthcare, factories and offices in 2D applications, and warehousing and logistics in 3D applications.

1.2 Product Features

Supports 2D positioning and cascaded gateways to reduce installation cost. The overview specifies accuracy down to 30 cm and describes improved accuracy and stability relative to Wi-Fi and Bluetooth positioning.

Maintenance: Phone mini-program configuration and centralized AC management.

Installation: Ceiling mounting reduces installation complexity and cost.

Power: Standard 48 V PoE carries power and return data over Ethernet, reducing cabling requirements.

1.3 Electrical Specifications

| Power supply | POE48V |
| --- | --- |
| Operating temperature | -20℃—70℃ |
| Operating humidity | 0 to 95%, non-condensing |
| Power consumption | Less than 3 W |

1.4 Technical Specifications

| Main MCU | 32-bit ARM M4 |
| --- | --- |
| Operating frequency | BLE：2.4GHz；UWB：6.0 to 6.5 GHz |
| Transmit power | BLE：8dBm UWB：-10dBm |
| Protocol | BLE5.0 802.15.4 |
| Receiver sensitivity | BLE:<-101dBm UWB: <-110dBm |
| Transmission range* | BLE：200m ； UWB：40m |
| Encryption | AES encryption supported |
| UWB channels | 5, 9 |
| UWB data rates | 850 kbps, 6.8 Mbps |
| Coverage radius | Greater than 30 m |
| Ranging accuracy | Greater than 30 cm |

Note: Transmission range is specified for line-of-sight conditions.

1.5 Physical Specifications

| Dimensions | 230*230*40mm |
| --- | --- |
| Color | White |
| Weight | 500g |
| Material | ABS |

1.6 Additional Specifications

| Network connection | 10/100 Mbps LAN port |
| --- | --- |
| Network addressing | DHCP or static IP |
| Network transport | UDP |
| Antenna | Internal antenna matrix |

1.7 Configuration Functions

| Destination IP | Domain name or IP address supported |
| --- | --- |
| Over-the-air update | OTA supported |
| Configuration method | Bluetooth mini program or AC |

| Network interface | RG-45 port (source designation) |
| --- | --- |

1.8 Interfaces

1.9 Operating Environment

1. Mount securely and horizontally on a ceiling where there is no significant movement, shock or vibration.

2. The environment must contain no gas or vapor that could damage the insulation or protective structure.

3. Use in a non-corrosive environment.

1.10 Packing List

| Accessory | JJ-U |
| --- | --- |
|  | UWB positioning station |
| Main unit | 1 unit |
| Certificate of conformity | 1 copy |
| Mounting screws and nuts | 1 set |
| Mounting backplate | 1 piece |