# Dual Mode LoRaWAN Gateway

English Product Documentation

An eight-channel LoRaWAN gateway built around an SX1302 concentrator, with integrated Bluetooth and multiple backhaul protocols for low-power IoT collection.

Jinju Intelligence (Beijing) Technology Co., Ltd. | zydc0915@gmail.com

## Selection Notes

4G is optional. Verify the regional frequency plan, terminal compatibility and shipping configuration for the project.

## Product Description

The JJ-IS40BL outdoor gateway supports low-power wide-area IoT, half-duplex and full-duplex communication, and 10/100 Mbps Ethernet cloud connectivity. Integrated Bluetooth and SX1302 baseband processing provide eight parallel uplink channels and one downlink channel for Bluetooth and LoRa terminals.

## Product Features

Full-duplex LoRa with eight receive channels and one downlink channel

Ethernet and optional 4G

UDP, TCP, MQTT, WebSocket and HTTP protocols

LoRaWAN standard protocol

LoRaWAN Class A/B/C

Transmit power up to 27 dBm and receiver sensitivity down to -142 dBm at SF12

Global LoRaWAN regional frequency plans and user-defined frequencies

Watchdog design for system stability

OTA firmware updates

Connection-recovery mechanisms designed to maintain terminal connectivity

ACServer management tools for configuration and administration

## Hardware Specifications

Bluetooth: BLE 5.0

Supports Bluetooth Long Range, connections, broadcast collection and other node functions.

LoRa concentrator: Eight channels plus an independent downlink channel, with maximum transmit power of 27 dBm and receiver sensitivity down to -142 dBm.

## Performance Specifications

## LoRa Specifications

| Parameter | Description |
| --- | --- |
| Output power | -1~27dBm |
| Input power | ＜10dBm |
| Bandwidth | 7.8~500kHz |
| Spreading factor | SF7~SF12 |
| Receiver sensitivity | -142dBm@SF12 BW125kHz |
| Harmonics | ＜-40dBm @ 1GHz |
| Frequency range | EU868 , AS923 , AU915 , KR920 ,RU864 ,IN865,US915 Global LoRaWAN regional frequency plans |
| Channels | Full duplex; eight LoRa receive channels and one LoRa downlink channel |

## Bluetooth Specifications

| Parameter | Description |
| --- | --- |
| Output power | -40~10dBm |
| Protocol | BLE 4, BLE 4.2 and BLE 5.0 supported |
| Data rate | Uncoded:1Mbps/2Mbps,Coded:125kbps(S=8)/500kbps(S=2) |
| Receiver sensitivity | <-97dBm |
| Coverage | >300M |

## Electrical Specifications

| Category | Description |
| --- | --- |
| Power supply | DC 12 V or PoE |
| Current | Max . 1A |
| Power consumption | ＜5W |

## Physical Characteristics

| Characteristic | Description |
| --- | --- |
| Dimensions | 216mm*216mm*70mm |
| Weight | Approximately 1.2 kg |
| Temperature | Operating: -20 to 70 degrees C; storage: -40 to 85 degrees C |
| Humidity | 5 to 95%, non-condensing |

## Interfaces

| Category | Description |
| --- | --- |
| Power connector | RJ45@poe |
| Ethernet port | RJ45 |
| LoRa antenna connector | N-type fiberglass antenna |
| Bluetooth antenna connector | N-type fiberglass antenna |