# E paper Location Badge

English Product Documentation

A 2.13-inch e-paper badge with Bluetooth, LoRa and GPS/BeiDou positioning. The source also lists AIS and solar charging specifications for projects needing visible identification and personnel location.

Jinju Intelligence (Beijing) Technology Co., Ltd. | zydc0915@gmail.com

Selection Notes

The typical four-nautical-mile AIS range assumes a receiving antenna ten meters above sea level. Confirm AIS and solar capabilities in the actual shipping configuration.

E Paper Personnel Location Badge Specifications

Electrical Specifications

| Operating voltage | 3.3V~4.2V |
| --- | --- |
| Charging supply | 5V1A |
| Maximum current | <800mA@3.3V |
| Listening current | <6Ma@3.3v |
| Bluetooth transmit current | <10Ma@3.3v |
| Sleep current | <30Ua@3.3V |
| Operating temperature | -20℃—60℃ |
| Operating humidity | 0 to 95%, non-condensing |

Solar Charging Specifications

| Operating voltage | 5V |
| --- | --- |
| Operating current | 50mA |
| Operating power | 0.3W |
| Open-circuit voltage | 6V |
| Short-circuit current | 66mA |
| Operating conditions | Unobstructed sunlight |

Display Specifications

| Screen size | 2.13In |
| --- | --- |
| Screen resolution | 122×250 pixel |
| Active display area | 23.7×48.5MM |

AIS Specifications

| Coverage | Typical range: 4 nautical miles with the receiving antenna 10 m above sea level |
| --- | --- |
| Positioning mode | GPS / BeiDou |
| Positioning accuracy | <5m (CEP50) |

Operating frequency

| LoRa frequency | 470MHz~510MHz |
| --- | --- |
| Bluetooth frequency | 2.4~2.485GHz |
| AIS frequency | AIS channel 1: 161.975 MHz AIS channel 2: 162.025 MHz |

Physical Specifications

| Dimensions | 104*70*14.5mm |
| --- | --- |
| Color | Red |
| Weight | 110g |
| Water resistance | IP68 |
| Housing material | ABS |

Additional Specifications

| Charging method | Magnetic charging |
| --- | --- |
| Charge protection | Overcharge protection |
| Low-voltage protection | Battery undervoltage and over-discharge protection |
| Emergency alarm | Supported |
| Motion and stationary detection | Supported |
| Vibration notification | Supported |
| Low-battery notification | Flashing LED |
| Firmware update | OTA supported |

Configuration Functions

| Filtering enable | Supported |
| --- | --- |
| Transmission frequency | Supported |
| Motion trigger | High, normal or low sensitivity |
| Transmission duration after becoming stationary | Supported |

Note: Configuration requires a phone supporting Bluetooth 4.2 or later.