# Outdoor Bluetooth LoRa Gateway

English Product Documentation

A cast-aluminum weather-resistant gateway combines Bluetooth and LoRa, with Ethernet backhaul and optional 4G variants. Collect personnel location data, environmental readings and equipment status.

Jinju Intelligence (Beijing) Technology Co., Ltd. | zydc0915@gmail.com

Selection Notes

Overview and specification-table figures differ for power, sensitivity and maximum range. Confirm any explosion-protection certification and its scope separately.

Product Specifications

Product: Smart Outdoor Bluetooth LoRa Positioning and Data Station

Model: JJ-IS40BLOP

Jinju Intelligence Beijing Technology Co Ltd

This document describes current general specifications and provides no guarantee of suitability for a particular use, function or merchantability. The company may change product functions, specifications, warranty information and this document without notice.

Product Introduction

Developed by Jinju Intelligence, this Bluetooth/LoRa gateway provides a front-end solution for wide-area IoT. Its software is optimized for common BLE and LoRa applications. DC-powered hardware supports eight physical RX channels and one TX channel. The overview specifies sensitivity down to -147 dBm and transmit power up to 27 dBm, with configurable power and frequency. Full-duplex operation uses an integrated cavity duplexer, a single antenna connector and a standard high-gain fiberglass antenna.

Applications include industrial positioning, temperature and humidity sensing, hazardous-chemical leak monitoring, equipment status, and low-power wide-area smart-city data collection for environmental, fire and traffic monitoring.

Product Images

Product Features

Coverage: Tested line-of-sight range up to 10 km depending on terminal type; typical coverage radius 1000 m.

Concurrency: Eight independent physical channels provide capacity for multiple devices.

Adaptive data rates: Simultaneous SF7 to SF12 reception without configuration.

Bidirectional communication: Full duplex with separate transmit and receive frequencies.

Flexible configuration: Adjustable transmit power and channel frequencies.

Receiver sensitivity: Down to -147 dBm in the overview.

Backhaul options: Ethernet or 4G.

Industrial design: The source describes water, dust and explosion protection, isolated interfaces and electromagnetic-interference resistance.

Specifications

| Physical Specifications | Dimensions | 216*216*70 |
| --- | --- | --- |
|  | Color | White |
|  | Weight | Main unit: 1.5 kg |
|  | Material | Cast aluminum |
|  | Ingress protection | IPX7 |
| Technical Specifications | Power supply | Waterproof RJ45 PoE connector; DC 12 V / 1 A adapter with 5.5 mm x 2.5 m connector dimensions as printed in the source; or DC 12 V / 1 A terminal block |
|  | Total power | 2W |
|  | Main MCU | 32-bit ARM M4 |
|  | Operating frequency | 2.4GHz |
|  | Transmit power | Bluetooth: 10 dBm; LoRa: 25 dBm |
|  | Protocol | Bluetooth 5.0; LoRa |
|  | Receiver sensitivity | BLE: < -100 dBm; LoRa: < -142 dBm |
|  | Transmission range* | BLE: 300 m line of sight; LoRa: 1000 m |
|  | Encryption | AES encryption supported |
|  | Network connection | 10/100 Mbps LAN or 4G for model JJ-IS40BL4GOP |
|  | Network addressing | DHCP or static IP |
|  | Network transport | UDP/MQTT |
|  | Antenna | External fiberglass antenna |
|  | Antenna connector | N-type female |
| Configuration Functions | Destination IP | Domain name or IP address supported |
|  | Over-the-air update | OTA supported |
|  | Configuration method | Bluetooth mini program or ACServer |
| Operating Environment | Operating temperature | -20℃—60℃ |
|  | Operating humidity | Up to 95% RH, non-condensing |

Packing List

| Accessory | JJ-IS40BOP | JJ-IS40BLOP | JJ-IS40B4GBOP | JJ-IS40BL4GBOP |
| --- | --- | --- | --- | --- |
|  | Outdoor Bluetooth | Outdoor Bluetooth LoRa | Outdoor Bluetooth 4G | Outdoor Bluetooth LoRa 4G |
| Main unit | √ | √ | √ | √ |
| Waterproof RJ45 Ethernet connector | √ | √ | √ | √ |
| Pole mounting clamps | √ | √ | √ | √ |
| Bluetooth fiberglass antenna, 2400 to 2500 MHz | √ | √ | √ | √ |
| LoRa fiberglass antenna, 470 to 510 MHz |  | √ |  | √ |
| 4G fiberglass antenna, 1700 to 2700 MHz |  |  | √ | √ |

Main unit; waterproof RJ45 Ethernet connector; pole mounting clamps

Quality Assurance

All units pass testing before shipment. A certificate of conformity and factory test report can be supplied.